Chapter 11 Solutions

- Using Shannon's formula, for 3 kHz bandwidth and 30 Kbps data rate, the S/N ratio is 1000.
The S/N in dB at normal operating condition is derived from 10(log101000) = 30 dB.  Every 3 dB decreases the S/N ratio by 1/2.
(a) C = 3*103(log2(1+500)) = 27 kbps
(b) C = 3*103(log2(1+250)) = 24 kbps.

- (a) The MIB objects to be monitored are ifInUcastpkts, ifInDiscards, ifOutUcastpkts, and ifOutDiscards.
(b) Packet loss ratio is given by the sum of the packet loss of input packets received and packet loss of output packets to be sent out.
% packet loss = 100((ifInUcastPkts/(ifInUcastpkts + ifInDiscards)) + (ifOutUcastPkts/(ifOutUcastPkts + ifOutDiscards)))

- (a) MoM NMS:		OSF, WSF
(b) Agent NMS		OSF, WSF
(c) Network element	NEF, QAF

- (a) The proxy server plays the function of a Media Device (MF).
(b) The interface of MF to the network manger is F, and to the network elements is QX.

- The Agent NMS could be treated as an OSF or as an MF.  Solutions for both cases are given below. 
						Reference Point	Interface
Between MoM NMS and Agent NMS			x			X
Between MoM NMS and Workstation			f			F
Between Agent NMS (MF) and NE			qx			QX
Between Agent NMS (OSF) and NE			q3			Q3
Between Agent NMS (OSF/MF) and Workstation	f			F

- Compare Figure 9.10 with Figure 11.4 / Figure 11.9.

M1 interface is between end user (network element) and network management system (operations system) and hence is either Q3 (TMN) or Qx (non-TMN) interface.  So are M2 and M4 interfaces

M3 and M5 interfaces go across different administrative boundaries and hence X interfaces.  Public and Private UNI and BICI also belong to this category.

Each workstation associated with NMSs interface with the respective NMS via an F interface.

- The following table compares the services of CMISE and SNMPv1

| CMISE Services | SNMPv1 Services |
| --- | --- |
| M-EVENT-REPORT | trap |
| Multiple responses | Not supported |
| M-GET | get or get-next |
| M-SET (confirmed / unconfirmed) | set-request (confirmed) |
| M-ACTION (confirmed / unconfirmed) | set-request (confirmed) |
| M-CREATE | Not supported |
| M-DELETE | Not supported |
| M-CANCEL-GET | Not supported |

- The following table compares the services of CMISE and SNMPv2

| CMISE Services | SNMPv2 Services |
| --- | --- |
| M-EVENT-REPORT | snmpV2-trap |
| Multiple responses | No direct equivalence; partly accomplished using get-bulk-request |
| M-GET | get or get-next, inform-request |
| M-SET (confirmed / unconfirmed) | set-request (confirmed) |
| M-ACTION (confirmed / unconfirmed) | set-request (confirmed) |
| M-CREATE | set-request (create using row status) -limited to create additional row in a table |
| M-DELETE | set-request (delete using row status) -limited to deleting row in a table |
| M-CANCEL-GET | Not supported |

- CMIP Object Identifier:
atmfM4CmipView OBJECT IDENTIFIER ::= {1.3.6.1.4.`.353.atmForumNetworkManagement(5).atmfM4(1).1}

SNMP Object Identifier:
atmfM4SnmpNEView OBJECT IDENTIFIER ::=
{1.3.6.1.4.`.353.atmForumNetworkManagement(5).atmfM4(1).3}

- Using af-nm-0027.0000 referenced in Table 9.3, we derived the following containment tree.
 				Figure for Exercise 10