Chapter 6 Solutions

- (a) 
hats  OBJECT-IDENTITY
 	STATUS		current
 	DESCRIPTION	"Hats is one of the class of products of 
				abcProducts".
 	::= {abcProducts 1}

(b) 
jacketQuantity  OBJECT IDENTITY
.	STATUS		current
 	DESCRIPTION	"Inventory value of a given size of the
 				jacket in the jackets table"
 	::= {jacketEntry 1}

ipAddrTable OBJECT-TYPE

SYNTAX		SEQUENCE OF IpAddrEntry

MAX-ACCESS	not-accessible

STATUS  		current

DESCRIPTION	"The table of addressing information relevant to this entity's  IP addresses."

::= {ip 20}

ipAddrEntry OBJECT-TYPE

SYNTAX		IpAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The addressing information for one of this entity's IP addresses."

INDEX   {ipAdEntAddr}

::= {ipAddrTable 1}

ipAdEntAddr OBJECT-TYPE

SYNTAX		IpAddress

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION	"The IP address to which this entry's addressing information pertains."

- (a) Base Table:

| ipAdEntAddr | … |
| --- | --- |
| 150.50.51.1 |  |
| 150.50.52.1 |  |
| 150.50.53.1 |  |
| 150.50.54.1 |  |

Augmented table:

| cardNumber | portNumber |
| --- | --- |
| 1 | 1 |
| 1 | 2 |
| 2 | 1 |
| 2 | 2 |

(b)
ipAddrTable	OBJECT-TYPE

SYNTAX		SEQUENCE OF IpAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The table …"

::= {ip 20}

ipAddrEntry	OBJECT-TYPE

SYNTAX		IpAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The addressing information …"

INDEX		{ipAdEntAddr}

;;= {ipAddrTable 1}

ipAdEntAddr	OBJECT-TYPE

SYNTAX		IpAddress

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION	"The IP address …"

;;= {ipAddrEntry 1}

ipAugAddrTable	OBJECT-TYPE

SYNTAX		SEQUENCE OF IpAugAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The augmented table to IP Address Table defining
 	 			board and port numbers"

::= {ipAug 1}

ipAugAddrEntry	OBJECT-TYPE

SYNTAX		IpAugAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The addressing information …"

AUGMENTS		{ ipAddrEntry }

::= {ipAugAddrTable 1}

cardNumber	OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION	"Interface card number"

;;= {ipAugAddrEntry 1}

portNumber	OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION	"Port number on interface card"

;;= {ipAugAddrEntry 2}

- (a) Base Table:

| Index | IP Address | Physical Address |
| --- | --- | --- |
| 3 | 172.46.41.1 | 00:00:0c:35:C1:D2 |
| 4 | 172.46.42.1 | 00:00:0c:35:C1:D3 |
| 5 | 172.46.43.1 | 00:00:0c:35:C1:D4 |
| 6 | 172.46.44.1 | 00:00:0c:35:C1:D5 |
| 2 | 172.46.63.1 | 00:00:0c:35:C1:D1 |
| 7 | 172.46.165.1 | 00:00:0c:35:C1:D8 |
| 1 | 172.46.252.1 | 00:00:0c:35:C1:D0 |

Augmented Table:

| intType | intNumber | portNumber |
| --- | --- | --- |
| 6 | 0 | 2 |
| 6 | 0 | 3 |
| 6 | 0 | 4 |
| 6 | 0 | 5 |
| 6 | 0 | 1 |
| 15 | 1 | 0 |
| 6 | 0 | 0 |

(b)
atTable	OBJECT-TYPE

SYNTAX		SEQUENCE OF atEntry

MAX-ACCESS	not-accessible

STATUS		deprecated

DESCRIPTION	"The table …"

::= {ip 20}

atEntry	OBJECT-TYPE

SYNTAX		IpAddrEntry

MAX-ACCESS	not-accessible

STATUS		deprecated

DESCRIPTION	"The addressing information …"

INDEX		{atIfIndex, atNetAddress}

::= {atTable 1}

IpAddrEntry::= 		SEQUENCE {

atIfIndex		INTEGER

atPhysAddress	Phys Address

atNetAddress	IP Address}

atIfIndex	OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-only

STATUS		deprecated

DESCRIPTION	"The interface…"

::= { atEntry 1 }

atPhysAddress	OBJECT-TYPE

SYNTAX		PhysAddress

MAX-ACCESS	read-only

STATUS		deprecated

DESCRIPTION	"The media-dependent…"

::= {atEntry 2}

atNetAddress	OBJECT-TYPE

SYNTAX		IpAddress

MAX-ACCESS	read-only

STATUS		deprecated

DESCRIPTION	"The Network address…"

::= {atEntry 3}

atAugTable	OBJECT-TYPE

SYNTAX		SEQUENCE OF atAugEntry

MAX-ACCESS	not-accessible

STATUS		deprecated

DESCRIPTION	"The table …"

::= {atAug 1}

atAugEntry	OBJECT-TYPE

SYNTAX		IpAugAddrEntry

MAX-ACCESS	not-accessible

STATUS		deprecated

DESCRIPTION	"The addressing information …"

AUGMENTS		{atEntry}

::= {atAugTable 1}

IpAugEntry::= 		SEQUENCE {

intType		INTEGER

intNumber		INTEGER

portNumber		INTEGER }

intType	OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION	"Interface card type.  Same as ifType in Interfaces 
 				group"

::= {atAugEntry 1}

intNumber	OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-only

STATUS		deprecated

DESCRIPTION	"Interface card number"

::= {atAugEntry 2}

portNumber	OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-only

STATUS		deprecated

DESCRIPTION	"Port number on interface card"

::= {atAugEntry 3}

- (a) Base Table:

| IntAdEntAddr | … |
| --- | --- |
| 150.50.51.1 |  |
| 150.50.52.1 |  |
| 150.50.53.1 |  |
| 150.50.54.1 |  |

Dependent table:

| CardNumber | portNumber |
| --- | --- |
| 1 | 1 |
| 1 | 2 |

(b)
ipAddrTable	OBJECT-TYPE

SYNTAX		SEQUENCE OF IpAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The table …"

::= {ip 20}

ipAddrEntry	OBJECT-TYPE

SYNTAX		IpAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The addressing information …"

INDEX		( ipAdEntAddr  }

::= {ipAddrTable 1}

ipAdEntAddr	OBJECT-TYPE

SYNTAX		IpAddress

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION	"The IP address …"

::= {ipAddrEntry 1}

ipDepAddrTable	OBJECT-TYPE

SYNTAX		SEQUENCE OF IpAugAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The augmented table to IP Address Table defining
 	 			board and port numbers"

::= {ipDep 1}

ipDepAddrEntry	OBJECT-TYPE

SYNTAX		IpDepAddrEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION	"The addressing information …"

INDEX		{ipAdEntAddr, cardNumber, portNumber}

::= {ipDepAddrTable 1}

IpDepAddrEntry ::= SEQUENCE {

IntNumber		INTEGER

portNumber		INTEGER}

intNumber	OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION	"Interface card number"

::= {ipDepAddrEntry 1}

portNumber	OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION	"Port number on interface card"

::= {ipDepAddrEntry 2}

- (a)
 				Figure for Exercise 6(a)
(b)
invTable	OBJECT-TYPE

SYNTAX		SEQUENCE OF InvEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"Inventory table."

::= {corp 100}

invEntry	OBJECT-TYPE

SYNTAX		InvEntry

MAX-ACCESS	not-accessible

STATUS		current

DESCRIPTION

"A conceptual row in the inventory table."

INDEX   { invNumber }

::= {invTable 1}

InvEntry ::= SEQUENCE {

invStatus	RowStatus

invNumber	INTEGER

make		DisplayString

model		DisplayString

serNumber	DisplayString}

invStatus		OBJECT-TYPE

SYNTAX		RowStatus

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"Status of the row"

::= {invEntry 1}

invNumber	OBJECT-TYPE

SYNTAX  		INTEGER

MAX-ACCESS	non-accessible

STATUS		current

DESCRIPTION

"Inventory number."

::= {invEntry 2}

make	OBJECT-TYPE

SYNTAX		DisplayString

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"Maker of the equipment"

::= {invEntry 3}

model	OBJECT-TYPE

SYNTAX		DisplayString

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"Model of the equipment"

::= {invEntry 4}

serNumber	OBJECT-TYPE

SYNTAX		DisplayString

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"Serial number of the equipment"

::= {invEntry 5}

- (a)
 						Figure for Exercise 7(a)
(b)
 						Figure for Exercise 7(b)

- Figure for Exercise 8

- atGroup		OBJECT-GROUP
 	OBJECTS		{atTable}
 	STATUS		deprecated
 	DESCRIPTION	"The atGroup is the Address Translation group containing the aggregate object of network to physical addresses."

- The rows will be retrieved in the lexicographic order based on the index field as shown below.  The second lexicographic sorting based on IP address (Network address) does not matter in this case.

| Index atIfIndex | Physical Address atPhysAddress | IP Address atNetAddress |
| --- | --- | --- |
| 1 | 00:00:0C:35:C1:D0 | 172.46.252.1 |
| 2 | 00:00:0C:35:C1:D1 | 172.46.63.1 |
| 3 | 00:00:0C:35:C1:D2 | 172.46.41.1 |
| 4 | 00:00:0C:35:C1:D3 | 172.46.42.1 |
| 5 | 00:00:0C:35:C1:D4 | 172.46.43.1 |
| 6 | 00:00:0C:35:C1:D5 | 172.46.44.1 |
| 7 | 00:00:0C:35:C1:D8 | 172.46.165.1 |

(a)
 					Figure for Exercise 10(a)
(b)
 					Figure for Exercise 10(b)

(c) Since we know the number of rows in this exercise, we can retrieve all the data using one get-bulk-request and response.  We need seven pairs of exchanges for the get-next-request.

- SNMPv2 Trap PDU is as shown in Figure 6.43

 					Figure for Exercise 11